### SECTION A: RESPONDENT INFORMATION

- Position/Job Role: ( ) Machine Operator (✓) Maintenance Technician ( ) Mechanical Engineer ( ) Electrical/Electronic Engineer ( ) Production Engineer ( ) Maintenance Supervisor ( ) Plant Manager ( ) Other:

- Years of experience: ( ) Less than 1 year ( ) 1–3 years (✓) 4–6 years ( ) 7–10 years ( ) More than 10 years

- Department/Unit: Process and Maintenance centrifugal Station

- Type of industry/company: ( ) Manufacturing (✓) Food Processing ( ) Chemical ( ) Pharmaceutical ( ) Textile ( ) Automotive ( ) Construction ( ) Oil & Gas ( ) Printing ( ) Other:

- How often do you interact with the machine? (✓) Daily ( ) Several times a week ( ) Weekly ( ) Occasionally

### SECTION B: MACHINE IDENTIFICATION AND GENERAL INFORMATION

- Machine name/type: Batch Sugar Centrifugal machine (separates sugar crystal from molasses)

- Machine identification/asset number: SC-003 (Sample)

- Manufacturer: /xample Sugar Machinery Ltd

- Model number: BC-1250

- Serial number (if applicable): SM-2013-0558

- Date/year of manufacture: 2013

- Date/year machine was installed/commissioned: 2014

- Age: ( ) Less than 2 years ( ) 2–5 years ( ) 6–10 years (✓) 11–15 years ( ) More than 15 years

- Locally manufactured or imported? ( ) Locally manufactured (✓) Imported ( ) Don't know

- Main function of the machine: Spin massecuite in a basket so sugar crystal are separated from molasses, then washed and discharges the sugar.

- Normal operating capacity: About 1,000 kg of sugar per cycle in about 3 minutes per cycle.

- Normal operating hours per day: ( ) <4 ( ) 4–8 ( ) 9–12 ( ) 13–16 (✓) >16

- Operating days per week: ( ) 1–2 ( ) 3–4 ( ) 5 (✓) 6

### SECTION C: MACHINE OPERATING PARAMETERS

Please indicate the important parameters normally monitored while the machine is operating.

| Parameter | Monitored? | How measured? |
| --- | --- | --- |
| Temperature | (✓) Yes ( ) No | Displayed bearing temperature gauge and motor temperature display |
| Vibration | (✓) Yes ( ) No | ffandheld vibration meter once per shift |
| Pressure | (✓) Yes ( ) No | Lubrication line pressure gauge |
| Speed/RPM | (✓) Yes ( ) No | Basket speed from drive controller |
| Voltage | (✓) Yes ( ) No | Panel voltmeter |
| Current | (✓) Yes ( ) No | Drive motor ammeter |
| Power consumption | ( ) Yes (✓) No | Only on the monthly electricity bill |

| Parameter | Monitored? | How measured? |
| --- | --- | --- |
| Oil level | (✓) Yes ( ) No | Gearbox and bearing oil sight glass |
| Oil pressure | ( ) Yes (✓) No |  |
| Fuel consumption | ( ) Yes (✓) No |  |
| Hydraulic pressure | ( ) Yes (✓) No |  |
| Noise/sound | ( ) Yes (✓) No | By ear only (operator experience) |
| Load | (✓) Yes ( ) No | Motor current during loading |
| Flow rate | ( ) Yes (✓) No |  |
| Other | (✓) Yes ( ) No | Cycle time, wash water and steam usage |

- Normal operating temperature: 55 °C (bearing)

- Temperature at which abnormal behaviour begins: 80 °C

- Normal operating speed/RPM: 1,200 RPM (basket at full speed)

- Normal operating pressure: Lubrication line about 2 bar

- Normal vibration level, if known: About 3 mm/s at the bearing housing

- Other important machine parameters: Cycle time about 3 minutes, basket load, wash water volume, sugar moisture

- Are parameters recorded electronically? (✓) Yes ( ) No

- If yes: (✓) Sensors (✓) PLC ( ) SCADA ( ) Computer software ( ) Mobile device ( ) Manual entry ( ) Other:

### SECTION D: MACHINE MAINTENANCE PRACTICES

- Main maintenance type: (✓) Preventive ( ) Corrective ( ) Predictive ( ) Breakdown ( ) Condition-based ( ) Combination

- Routine maintenance frequency: ( ) Daily (✓) Weekly ( ) Monthly ( ) Quarterly ( ) Every 6 months ( ) Annually ( ) Based on condition

- Recommended service interval: /very 2,000 operating hours (bearing), oil every 6 months

- Date of last major service: 25/07/2026

- Date of next scheduled service: 25/01/2027

### SECTION G (continued): SOUND

- Describe the sound immediately before failure: Loud rhythmic knocking and a grinding noise from the bearing

- Does the abnormal sound increase gradually? (✓) Yes ( ) No ( ) Sometimes

- Can an experienced operator identify a fault from sound? (✓) Yes ( ) No ( ) Sometimes

- Usefulness of automatic sound monitoring: (✓) Very useful ( ) Useful ( ) Moderate ( ) Slight ( ) Not useful

### SECTION H: VISUAL AND PHYSICAL MACHINE CHANGES

| Observation | Never | Rarely | Sometimes | Often | Always |
| --- | --- | --- | --- | --- | --- |
| Excessive vibration | ( ) | ( ) | ( ) | (✓) | ( ) |
| Increased temperature | ( ) | ( ) | ( ) | (✓) | ( ) |
| Oil leakage | ( ) | ( ) | (✓) | ( ) | ( ) |
| Smoke | (✓) | ( ) | ( ) | ( ) | ( ) |
| Burning smell | ( ) | (✓) | ( ) | ( ) | ( ) |

| Observation | Never | Rarely | Sometimes | Often | Always |
| --- | --- | --- | --- | --- | --- |
| Unusual noise | ( ) | ( ) | (✓) | ( ) | ( ) |
| Reduced speed | ( ) | ( ) | ( ) | (✓) | ( ) |
| Reduced output | ( ) | ( ) | (✓) | ( ) | ( ) |
| Increased energy consumption | ( ) | ( ) | (✓) | ( ) | ( ) |
| Irregular movement | ( ) | ( ) | (✓) | ( ) | ( ) |
| Frequent machine stoppage | ( ) | ( ) | (✓) | ( ) | ( ) |

### SECTION I: FAULT DETECTION AND DIAGNOSIS

- Fault detection method: (✓) Operator observation ( ) Alarm system ( ) Sensor ( ) PLC/SCADA ( ) Routine inspection ( ) Machine stops ( ) Technician diagnosis ( ) Other:

- Fault cause determination: (✓) Operator experience ( ) Maintenance manual ( ) Technician inspection ( ) Diagnostic software ( ) Sensor readings ( ) Trial and error ( ) Manufacturer support ( ) Other:

- Normal diagnosis time: ( ) <30 minutes ( ) 30 min–1 hour (✓) 1–3 hours ( ) 4–8 hours ( ) >8 hours

- Challenges: (✓) Lack of sensors ( ) Lack of maintenance records ( ) Lack of skilled personnel ( ) Machine complexity ( ) Lack of diagnostic equipment ( ) Lack of historical data ( ) Poor documentation ( ) Other:

### SECTION J: USE OF SENSORS AND SMART MONITORING

- Does the machine have condition-monitoring sensors? (✓) Yes ( ) No ( ) Don't know

- Sensor types: (✓) Temperature ( ) Vibration ( ) Pressure ( ) Current ( ) Voltage ( ) Speed ( ) Sound/acoustic ( ) Humidity ( ) Oil condition ( ) Other:

- Are sensor readings stored for future analysis? ( ) Yes (✓) No ( ) Don't know

- Measurement frequency: (✓) Continuously ( ) Every second ( ) Every minute ( ) Every 5–10 minutes ( ) Hourly ( ) Daily ( ) Manual

- Would the company be willing to use sensors for continuous machine-health monitoring? (✓) Yes ( ) No ( ) Maybe

### SECTION K: PREDICTIVE MAINTENANCE EXPERT SYSTEM

- Does the company currently use a predictive maintenance system? ( ) Yes (✓) No ( ) Don't know

- Would an intelligent system that predicts possible failure be useful? (✓) Very useful ( ) Useful ( ) Moderate ( ) Slight ( ) Not useful

- Information the system should monitor: (✓) Temperature (✓) Vibration ( ) Pressure (✓) Current ( ) Voltage (✓)

Speed (✓) Machine sound (✓) Operating hours ( ) Maintenance history ( ) Machine age (✓) Fault history (✓) Production output

- Preferred alert: ( ) Computer warning ( ) Mobile notification ( ) SMS ( ) Sound alarm ( ) Dashboard ( ) Email (✓) Combination

- Preferred warning time: ( ) Minutes ( ) Hours ( ) 1 day (✓) Several days ( ) Several weeks

- Would you trust automated maintenance recommendations? ( ) Yes ( ) No (✓) Only with human confirmation

- Final maintenance decision should be made by: ( ) System ( ) Maintenance personnel ( ) Operator (✓) System recommendation + human approval

### SECTION L: EDGE-BASED MONITORING

- Would the company prefer machine data to be processed locally near the machine? (✓) Yes ( ) No ( ) Not sure

- Advantages: (✓) Faster fault detection (✓) Reduced internet dependence ( ) Improved data privacy (✓) Faster response (✓) Reduced data transmission (✓) Continuous monitoring ( ) Other:

- Internet/network reliability: ( ) Very reliable ( ) Reliable (✓) Sometimes unreliable ( ) Frequently unreliable ( ) Not available

- Should the system continue operating when internet connectivity is unavailable? (✓) Yes ( ) No ( ) Not sure

### SECTION M: MAINTENANCE RECORD DATA

| Date | Parameter / Observa | tFioanult | Suspected Cause | Maintenance Done | Parts Replaced | Downtime |
| --- | --- | --- | --- | --- | --- | --- |
| 18/09/26 | ffeavy shaking at full speed; bearing at 78 °C | Basket vibration | Worn bearing; uneven sugar build-up | Cleaned basket; replaced bearing and rebalanced | Main bearing | 4 hours |
| 06/08/26 | Longer cycle time; motor current high | Slow acce leration | Slipping drive belts | Belts replaced and tensioned | Drive belts (x4) | 3 hours |
| 14/06/26 | Sugar too wet after spinning | Poor sepa ration | Worn basket screen | Screen replaced | Basket screen | 6 hours |

### SECTION N: OVERALL ASSESSMENT

Scale: SA = Strongly Agree; A = Agree; U = Undecided; D = Disagree; SD = Strongly Disagree.

| Statement | SA | A | U | D | SD |
| --- | --- | --- | --- | --- | --- |
| Machine failures can often be detected before complete breakdown. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Machine operating parameters are useful for predicting faults. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Machine sound can provide information about machine health. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Maintenance history is important for predicting future faults. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Preventive maintenance reduces unexpected machine breakdown. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Continuous machine monitoring would improve maintenance activities. | (✓) | ( ) | ( ) | ( ) | ( ) |
| An intelligent predictive maintenance system would reduce machine downtime. | ( ) | (✓) | ( ) | ( ) | ( ) |
| Sensor-based monitoring would improve fault detection. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Automated alerts would help maintenance personnel respond faster. | ( ) | (✓) | ( ) | ( ) | ( ) |
| An edge-based system would be useful in an industrial environment. | ( ) | (✓) | ( ) | ( ) | ( ) |

### SECTION O: OPEN-ENDED QUESTIONS

- What are the most common causes of machine failure in your company?

## Bearing wear, basket unbalance, worn belts, crystal and molasses build-up, and poor lubrication.

- What changes do you normally observe shortly before a machine develops a serious fault?

## Vibration rises, bearing temperature climbs, the machine shakes at full speed and cycle time gets longer.

- Are there particular sounds that indicate a developing fault? Please describe them.

## A rhythmic knock or grind from the bearing, and a louder whine while the basket speeds up.

- What machine parameters are most useful for predicting failure?

## Bearing vibration, bearing temperature, motor current, basket speed and cycle time.

- What challenges do you face with the current maintenance system?

## Vibration is only checked by hand once per shift, so fast-developing faults are missed.

- What features would you like to see in a predictive maintenance system?

## Continuous vibration monitoring, early warnings several days ahead, and alerts on phone.

- Other information that may help in developing the expert system:

## Sugar load and moisture affect balance and current, so the system should take them into account.