SECTION A: RESPONDENT INFORMATION

| 1. Position/Job Role | Maintenance Supervisor (sample response) |
| --- | --- |
| 2. Years of experience | 7–10 years |
| 3. Department/Unit | Packaging / Bottling Hall |
| 4. Type of industry/company | Food Processing / Brewing and Beverage Manufacturing |
| 5. How often do you interact with the machine? | Daily |

SECTION B: MACHINE IDENTIFICATION AND GENERAL INFORMATION

| 1. Machine name/type | Bottle Washer |
| --- | --- |
| 2. Machine identification/asset number | NB-PK-WSH-01 (sample asset ID – verify from plant asset register) |
| 3. Manufacturer | Imported bottling-line OEM (sample) |
| 4. Model number | BW-36 (sample) |
| 5. Serial number | SN-2012-0307 (sample) |
| 6. Date/year of manufacture | 2012 |
| 7. Date/year installed/commissioned | 2013 |
| 8. Age | 11–15 years |
| 9. Locally manufactured or imported? | Imported |
| 10. Main function | Cleans, rinses and sanitizes returnable bottles with hot caustic solution before filling. |
| 11. Normal operating capacity | About 60,000 bottles/hr (representative; verify plant record) |
| 12. Normal operating hours per day | 13–16 hours (representative production schedule; verify plant record) |
| 13. Operating days per week | 6 days (representative; verify plant schedule) |

SECTION C: MACHINE OPERATING PARAMETERS

| Parameter | Monitored? | How measured / typical observation |
| --- | --- | --- |
| Temperature | Yes | Caustic bath temperature probe; read on panel/PLC |
| Vibration | No | Not monitored routinely; checked by touch/listening only |
| Pressure | Yes | Spray-water pressure gauge |
| Speed/RPM | Yes | Drive panel / HMI |
| Voltage | Yes | Panel voltmeter / PLC monitoring |
| Current | Yes | Panel ammeter / drive current display |
| Power consumption | Yes | Energy meter / drive data |
| Oil level | Yes | Sight glass on gearbox/crankcase |
| Oil pressure | No | Not monitored on this machine |
| Fuel consumption | No | Not applicable – electric machine |
| Hydraulic pressure | No | No hydraulic subsystem fitted |
| Noise/sound | No | Not monitored; operator listening only |
| Load | No | Not monitored on this machine |
| Flow rate | Yes | Caustic and rinse-water flow meters |
| Other | Yes | Caustic concentration, bath temperature, bottle cleanliness |

| 2. Normal operating temperature | About 80 °C (caustic bath temperature probe) |
| --- | --- |
| 3. Temperature at which abnormal behaviour begins | About 90 °C |
| 4. Normal operating speed/RPM | 1,450 RPM (drive motor; about 60,000 bottles/hr) |
| 5. Normal operating pressure | About 3 bar (spray-water pressure) |
| 6. Normal vibration level, if known | Not measured routinely; about 4 mm/s RMS expected for a healthy machine – investigate a sustained rise above baseline. |
| 7. Other important machine parameters | Caustic concentration, bath temperature, bottle cleanliness. |
| 8. Are parameters recorded electronically? | Yes – PLC/HMI. |
| 9. If yes | Sensors + PLC/HMI; logbook for major observations. |

SECTION D: MACHINE MAINTENANCE PRACTICES

| 1. Main maintenance type | Combination |
| --- | --- |
| 2. Routine maintenance frequency | Daily; major service per manufacturer/plant schedule |
| 3. Recommended service interval | 2,000 operating hours |
| 4. Date of last major service | 28 August 2026 (sample) |
| 5. Date of next scheduled service | 28 February 2027 (sample) |
| 6. Maintenance activities | Chain and carrier inspection; nozzle cleaning; caustic heater descaling; pump bearing checks; lubrication. |
| 7. Who performs maintenance? | Company maintenance personnel |
| 8. Is a maintenance logbook kept? | Yes |
| 9. Are previous machine faults recorded? | Often |
| 10. Usefulness of maintenance history for diagnosis | Useful |

SECTION E: MACHINE FAILURE AND FAULT HISTORY

| 1. Has the machine experienced a major failure? | Yes (sample response) |
| --- | --- |
| 2. Fault frequency | Frequently (representative response; verify actual maintenance records) |
| 3. Most common faults | Chain wear; nozzle blockage; caustic heater scale; pump bearing wear. |
| 4. Component that fails most frequently | Chain and carriers |
| 5. Date of most recent failure | 04 October 2026 (sample) |
| 6. Cause of most recent failure, if known | Chain wear and misalignment. |
| 7. Downtime after failure | 5–8 hours |
| 8. Action taken to restore the machine | Chain links replaced, carriers aligned, lubricated. |
| 9. Was the failure predictable? | Yes |
| 10. Warning signs observed | Noise from chain, carrier misalignment, poor bottle cleanliness. |

SECTION F: MACHINE BEHAVIOUR BEFORE FAILURE

| 1. Does the machine show warning signs before a fault? | Often |
| --- | --- |
| 2. Warning signs observed | Unusual noise; irregular movement; reduced output; error message. |
| 3. How long before failure do warning signs appear? | Days |
| 4. Earliest indication of a developing fault | Chain rattle and carrier misalignment. |
| 5. Most reliable indication of impending failure | Repeated carrier jams with chain noise. |
| 6. Can an experienced operator detect an upcoming fault through observation? | Yes |
| 7. What does the operator normally observe? | Broken bottles, noise, jams, dirty bottles. |

SECTION G: MACHINE SOUND AND NOISE BEHAVIOUR

| 1. Does the machine produce a different sound when developing a fault? | Yes |
| --- | --- |
| 2. Abnormal sound | Rattling; clicking; squealing |
| 3. Component producing abnormal sound | Chain drive and carriers |
| 4. Describe the normal machine sound | Regular chain rhythm and water spray. |
| 5. Describe the sound immediately before failure | Loud chain rattle and squeal. |
| 6. Does the abnormal sound increase gradually? | Yes |
| 7. Can an experienced operator identify a fault from sound? | Yes |
| 8. Usefulness of automatic sound monitoring | Very useful |

SECTION H: VISUAL AND PHYSICAL MACHINE CHANGES

| Observation | Frequency |
| --- | --- |
| Excessive vibration | Sometimes |
| Increased temperature | Sometimes |
| Oil leakage | Rarely |
| Smoke | Never |
| Burning smell | Rarely |
| Unusual noise | Often |
| Reduced speed | Sometimes |
| Reduced output | Often |
| Increased energy consumption | Sometimes |
| Irregular movement | Often |
| Frequent machine stoppage | Often |

SECTION I: FAULT DETECTION AND DIAGNOSIS

| 1. Fault detection method | Operator observation; alarm system; PLC/HMI; machine stops |
| --- | --- |
| 2. Fault cause determination | Operator experience; technician inspection; maintenance manual |
| 3. Normal diagnosis time | 30 min–1 hour |
| 4. Challenges | Machine complexity; lack of historical data. |

SECTION J: USE OF SENSORS AND SMART MONITORING

| 1. Does the machine have condition-monitoring sensors? | Yes – bath temperature, pressure, flow |
| --- | --- |
| 2. Sensor types | Temperature; pressure; flow; current; speed |
| 3. Are sensor readings stored for future analysis? | Yes – PLC/HMI where installed |
| 4. Measurement frequency | Every second |
| 5. Would the company be willing to use sensors for continuous machine-health monitoring? | Yes |

SECTION K: PREDICTIVE MAINTENANCE EXPERT SYSTEM

| 1. Does the company currently use a predictive maintenance system? | No |
| --- | --- |
| 2. Would an intelligent system that predicts possible failure be useful? | Very useful |
| 3. Information the system should monitor | Temperature; pressure; current; machine sound; fault history. |
| 4. Preferred alert | Combination (dashboard, SMS, sound alarm) |
| 5. Preferred warning time | Hours to several days |
| 6. Would you trust automated maintenance recommendations? | Only with human confirmation |
| 7. Final maintenance decision should be made by | System recommendation + human approval |

SECTION L: EDGE-BASED MONITORING

| 1. Prefer data processed locally near machine? | Yes |
| --- | --- |
| 2. Advantages | Faster response; reduced internet dependence. |
| 3. Internet/network reliability | Sometimes unreliable |
| 4. Should the system continue when internet is unavailable? | Yes – local edge processing should continue and synchronise data when connectivity returns. |

SECTION M: MAINTENANCE RECORD DATA – SAMPLE ENTRIES

| Date | Parameter / Observation | Fault Suspected | Cause | Maintenance Done | Parts Replaced | Downtime |
| --- | --- | --- | --- | --- | --- | --- |
| 04 / 10 / 2026 | Chain rattle, carrier jams | Chain wear | Wear and misalignment | Chain links replaced | Chain links | 6 h |
| 17 / 07 / 2026 | Dirty bottles at outlet | Nozzle blockage | Scale build-up | Nozzles descaled | Nozzles | 3 h |

SECTION N: OVERALL ASSESSMENT

Scale: SA = Strongly Agree; A = Agree; U = Undecided; D = Disagree; SD = Strongly Disagree.

| Statement | Rating |
| --- | --- |
| Machine failures can often be detected before complete breakdown. | SA |
| Machine operating parameters are useful for predicting faults. | SA |
| Machine sound can provide information about machine health. | SA |
| Maintenance history is important for predicting future faults. | SA |
| Preventive maintenance reduces unexpected machine breakdown. | SA |
| Continuous machine monitoring would improve maintenance activities. | SA |
| An intelligent predictive maintenance system would reduce machine downtime. | SA |
| Sensor-based monitoring would improve fault detection. | A |
| Automated alerts would help maintenance personnel respond faster. | SA |
| An edge-based system would be useful in an industrial environment. | A |

SECTION O: OPEN-ENDED QUESTIONS

| 1. Most common causes of machine failure | Chain wear, nozzle blockage, caustic heater scale, pump bearing wear. |
| --- | --- |
| 2. Changes observed before serious fault | Chain rattle, jams and dirty bottles increase. |
| 3. Sounds indicating developing fault | Rattling or squealing from the chain drive. |
| 4. Most useful parameters for predicting failure | Bath temperature, caustic concentration, chain current, vibration. |
| 5. Challenges with current maintenance system | Complex machine, manual checks. |
| 6. Desired predictive maintenance system features | Chain wear estimate, jam prediction, mobile alerts. |
| 7. Other information useful for the expert system | Bottle condition affects jam and breakage rates. |