#### SECTION A: RESPONDENT INFORMATION

- Position/Job Role: ( ) Machine Operator ( ) Maintenance Technician (✓) Mechanical Engineer ( ) Electrical/Electronic Engineer ( ) Production Engineer ( ) Maintenance Supervisor ( ) Plant Manager ( ) Other

- Years of experience: ( ) Less than 1 year ( ) 1–3 years ( ) 4–6 years ( ) 7–10 years (✓) More than 10 years

- Department/Unit: Cement Grinding department

- Type of industry/company: (✓) Manufacturing ( ) Food Processing ( ) Chemical ( ) Pharmaceutical ( ) Textile ( ) Automotive ( ) Construction ( ) Oil & Gas ( ) Printing ( ) Other

- How often do you interact with the machine? (✓) Daily ( ) Several times a week ( ) Weekly ( ) Occasionally

#### SECTION B: MACHINE IDENTIFICATION AND GENERAL INFORMATION

- Machine name/type: Cement Ball Mill (clinker grinding)

- Machine identification/asset number: CM-001 (Sample)

- Manufacturer: Example Heavy Industries Ltd (Sample)

- Model number: BM-3.2x13 (Sample)

- Serial number (if applicable): SN-2008-0215 (Sample)

- Date/year of manufacture: 2008

- Date/year machine was installed/commissioned: 2009

- Age: ( ) Less than 2 years ( ) 2–5 years ( ) 6–10 years ( ) 11–15 years (✓) More than 15 years

- Locally manufactured or imported? ( ) Locally manufactured (✓) Imported ( ) Don't know

- Main function of the machine: Grinds clinker and gypsum into fine cement powder

- Normal operating capacity: About 40 tonnes per hour

- Normal operating hours per day: ( ) <4 ( ) 4–8 ( ) 9–12 ( ) 13–16 (✓) >16

- Operating days per week: ( ) 1–2 ( ) 3–4 ( ) 5 (✓) 6 ( ) 7

#### SECTION C: MACHINE OPERATING PARAMETERS

- Please indicate the important parameters normally monitored while the machine is operating.

| Parameter | Monitored? | How measured? |
| --- | --- | --- |
| Temperature | (✓) Yes ( ) No | Bearing and Mill-outlet temperature sensors on SCADA |
| Vibration | (✓) Yes ( ) No | Handheld vibration meter on bearings |
| Pressure | (✓) Yes ( ) No | Lube oil pressure gauge |
| Speed/RPM | (✓) Yes ( ) No | Mill speed from drive controller |

| Voltage | (✓) Yes ( ) No | Panel voltmeter |
| --- | --- | --- |
| Current | (✓) Yes ( ) No | Main motor ammeter |
| Power consumption | (✓) Yes ( ) No | KWh meter |
| Oil level | (✓) Yes ( ) No | Bearing lube tank level |
| Oil pressure | (✓) Yes ( ) No | Lube oil pressure gauge |
| Fuel consumption | ( ) Yes (✓) No |  |
| Hydraulic pressure | (✓) Yes ( ) No | Hydraulic lubrication unit gauge |
| Noise/sound | (✓) Yes ( ) No | Electronic ear and operator listening |
| Load | (✓) Yes ( ) No | Mill load from motor current |
| Flow rate | (✓) Yes ( ) No | Weigh feeder (tonnes per hour) |
| Other | (✓) Yes ( ) No | Mill outlet temperature, cement fineness |

- Normal operating temperature: 110°C

- Temperature at which abnormal behaviour begins: 130°C

- Normal operating speed/RPM: 15 RPM

- Normal operating pressure: 3 bar

- Normal vibration level, if known: About 4 mm/s at the main

- Other important machine parameters: Feed rate 40t/h, Cement fineness

- Are parameters recorded electronically? (✓) Yes ( ) No

- If yes: (✓) Sensors ( ) PLC ( ) SCADA ( ) Computer software ( ) Mobile device ( ) Manual entry ( ) Other

#### SECTION D: MACHINE MAINTENANCE PRACTICES

- Main maintenance type: ( ) Preventive ( ) Corrective ( ) Predictive ( ) Breakdown ( ) Condition-based (✓)

Combination

- Routine maintenance frequency: (✓) Daily ( ) Weekly ( ) Monthly ( ) Quarterly ( ) Every 6 months ( ) Annually ( ) Based on condition

- Recommended service interval: Major Overhaul every 12 months

- Date of last major service: 10 / 03 / 2026

- Date of next scheduled service: 10 / 03 / 2027

- Maintenance activities: (✓) Lubrication ( ) Oil change ( ) Filter replacement ( ) Bearing inspection ( ) Belt inspection/replacement ( ) Electrical inspection ( ) Cleaning ( ) Calibration ( ) Tightening ( ) Replacement of worn parts ( ) PLC/software checks ( ) Other

- Who performs maintenance? ( ) Operator ( ) Company maintenance personnel ( ) External technician ( ) Manufacturer (✓) Combination

- Is a maintenance logbook kept? (✓) Yes ( ) No

- Are previous machine faults recorded? (✓) Always ( ) Often ( ) Sometimes ( ) Rarely ( ) Never

- Usefulness of maintenance history for diagnosis: (✓) Very useful ( ) Useful ( ) Moderate ( ) Slight ( ) Not useful

#### SECTION E: MACHINE FAILURE AND FAULT HISTORY
Has the machine experienced a major failure? (✓) Yes ( ) No ( ) Don't know

- Fault frequency: ( ) Very frequently ( ) Frequently (✓) Occasionally ( ) Rarely

- Most common faults: Bearing overheating, linear wear, lube pump failure

- Component that fails most frequently: Main bearing lubrication system and mill liners

- Date of most recent failure: 14 / 07 / 2026

- Cause of most recent failure, if known: The lubrication pump failed and the bearing temp rose.

- Downtime after failure: ( ) <1 hour ( ) 1–4 hours ( ) 5–8 hours ( ) 9–24 hours (✓) >24 hours

- Action taken to restore the machine: Replaced the pump, flushed the oil system and inspected the bearing.

- Was the failure predictable? (✓) Yes ( ) No ( ) Not sure

- Warning signs observed: Bearing temperature rising, oil pressure dropping, vibration and motor current increasing

#### SECTION F: MACHINE BEHAVIOUR BEFORE FAILURE

- Does the machine show warning signs before a fault? ( ) Always (✓) Often ( ) Sometimes ( ) Rarely ( ) Never

- Warning signs observed: ( ) Increased temperature ( ) Excessive vibration (✓) Unusual sound/noise ( ) Reduced speed ( ) Increased power consumption ( ) Increased current ( ) Pressure changes ( ) Smoke ( ) Burning smell ( ) Oil leakage ( ) Reduced production output ( ) Irregular movement ( ) Excessive heating ( ) Intermittent stopping ( ) Alarm light ( ) Error message/code ( ) Other

- How long before failure do warning signs appear? ( ) Minutes ( ) Hours ( ) 1 day (✓) Several days ( ) Weeks ( ) No fixed period

- Earliest indication of a developing fault: A slow rise in bearing temperature

- Most reliable indication of impending failure: Rising bearing temperature together with falling lube oil pressure

- Can an experienced operator detect an upcoming fault through observation? (✓) Yes ( ) No ( ) Sometimes

- What does the operator normally observe? SCADA trends, mill sound, bearing temperature and oil leaks

#### SECTION G: MACHINE SOUND AND NOISE BEHAVIOUR

- Does the machine produce a different sound when developing a fault? (✓) Yes ( ) No ( ) Sometimes ( ) Not sure

- Abnormal sound: ( ) Grinding (✓) Knocking ( ) Clicking ( ) Humming ( ) Screeching ( ) Squealing ( ) Rattling ( ) Whining ( ) Hissing ( ) Popping ( ) Increased general noise ( ) Other

- Component producing abnormal sound: Mill shell, Main bearings, girth gear, gearbox

- Describe the normal machine sound: A continuous, steady roar of grinding balls

- Describe the sound immediately before failure: Loud metallic knocking and a high whine from the bearing or gearbox

- Does the abnormal sound increase gradually? (✓) Yes ( ) No ( ) Sometimes

- Can an experienced operator identify a fault from sound? (✓) Yes ( ) No ( ) Sometimes

- Usefulness of automatic sound monitoring: (✓) Very useful ( ) Useful ( ) Moderate ( ) Slight ( ) Not useful

#### SECTION H: VISUAL AND PHYSICAL MACHINE CHANGES

| Observation | Never | Rarely | Sometimes | Often | Always |
| --- | --- | --- | --- | --- | --- |

| Excessive vibration | ( ) | ( ) | ( ) | (✓) | ( ) |
| --- | --- | --- | --- | --- | --- |
| Increased temperature | ( ) | ( ) | ( ) | (✓) | ( ) |
| Oil leakage | ( ) | ( ) | (✓) | ( ) | ( ) |
| Smoke | (✓) | ( ) | ( ) | ( ) | ( ) |
| Burning smell | ( ) | (✓) | ( ) | ( ) | ( ) |
| Unusual noise | ( ) | ( ) | ( ) | (✓) | ( ) |
| Reduced speed | ( ) | (✓) | ( ) | ( ) | ( ) |
| Reduced output | ( ) | ( ) | ( ) | (✓) | ( ) |
| Increased energy consumption | ( ) | ( ) | ( ) | (✓) | ( ) |
| Irregular movement | ( ) | (✓) | ( ) | ( ) | ( ) |
| Frequent machine stoppage | ( ) | ( ) | (✓) | ( ) | ( ) |

#### SECTION I: FAULT DETECTION AND DIAGNOSIS

- Fault detection method: (✓) Operator observation (✓) Alarm system (✓) Sensor (✓) PLC/SCADA (✓) Routine inspection (✓) Machine stops (✓) Technician diagnosis ( ) Other

- Fault cause determination: (✓) Operator experience (✓) Maintenance manual (✓) Technician inspection ( ) Diagnostic software (✓) Sensor readings ( ) Trial and error (✓) Manufacturer support ( ) Other

- Normal diagnosis time: ( ) <30 minutes ( ) 30 min–1 hour ( ) 1–3 hours (✓) 4–8 hours ( ) >8 hours

- Challenges: ( ) Lack of sensors ( ) Lack of maintenance records (✓) Lack of skilled personnel (✓) Machine complexity ( ) Lack of diagnostic equipment (✓) Lack of historical data ( ) Poor documentation ( ) Other

#### SECTION J: USE OF SENSORS AND SMART MONITORING

- Does the machine have condition-monitoring sensors? (✓) Yes ( ) No ( ) Don't know

- Sensor types: (✓) Temperature ( ) Vibration ( ) Pressure ( ) Current ( ) Voltage ( ) Speed ( ) Sound/acoustic ( ) Humidity ( ) Oil condition ( ) Other

- Are sensor readings stored for future analysis? (✓) Yes ( ) No ( ) Don't know

- Measurement frequency: ( ) Continuously ( ) Every second (✓) Every minute ( ) Every 5–10 minutes ( ) Hourly ( ) Daily ( ) Manual

- Would the company be willing to use sensors for continuous machine-health monitoring? (✓) Yes ( ) No ( ) Maybe

#### SECTION K: PREDICTIVE MAINTENANCE EXPERT SYSTEM

- Does the company currently use a predictive maintenance system? ( ) Yes (✓) No ( ) Don't know

- Would an intelligent system that predicts possible failure be useful? (✓) Very useful ( ) Useful ( ) Moderate ( ) Slight ( ) Not

- Information the system should monitor: (✓) Temperature (✓) Vibration (✓) Pressure (✓) Current (✓) Voltage (✓) Speed (✓) Machine sound (✓) Operating hours (✓) Maintenance history (✓) Machine age (✓) Fault history (✓) Production output

- Preferred alert: ( ) Computer warning ( ) Mobile notification ( ) SMS ( ) Sound alarm (✓) Dashboard ( ) Email ( ) Combination

- Preferred warning time: ( ) Minutes ( ) Hours ( ) 1 day ( ) Several days (✓) Several weeks

- Would you trust automated maintenance recommendations? ( ) Yes ( ) No (✓) Only with human confirmation

- Final maintenance decision should be made by: ( ) System ( ) Maintenance personnel ( ) Operator (✓) System recommendation + human approval

#### SECTION L: EDGE-BASED MONITORING

- Would the company prefer machine data to be processed locally near the machine? (✓) Yes ( ) No ( ) Not sure

- Advantages: (✓) Faster fault detection ( ) Reduced internet dependence (✓) Improved data privacy (✓) Faster response (✓) Reduced data transmission (✓) Continuous monitoring ( ) Other

- Internet/network reliability: ( ) Very reliable ( ) Reliable (✓) Sometimes unreliable ( ) Frequently unreliable ( ) Not available

- Should the system continue operating when internet connectivity is unavailable? (✓) Yes ( ) No ( ) Not sure

#### SECTION M: MAINTENANCE RECORD DATA

| Date | Parameter / Observation | Fault | Suspected Cause | Maintenance Done | Parts Replaced | Downtime |
| --- | --- | --- | --- | --- | --- | --- |
| 14/07/2 026 | Bearing temp rose 15°C over 4 days, oil pressure dropped | Bearing overheating | Failed lube pump | Pump replaced, lube oil flushed | Lube pump | 30 hours |
| 09/05/2 026 | Fineness off target, motor current up | Reduced output | Worn mill liners | Liners inspected and partly replaced | Liner plates | 20 hours |
| 20/02/2 026 | Knocking from girth gear | Gear noise | Low gear lubrication | Re-lubricated, alignment checked | No / NIL | 4 hours |

#### SECTION N: OVERALL ASSESSMENT

Scale: SA = Strongly Agree; A = Agree; U = Undecided; D = Disagree; SD = Strongly Disagree.

| Statement | SA | A | U | D | SD |
| --- | --- | --- | --- | --- | --- |
| Machine failures can often be detected before complete breakdown. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Machine operating parameters are useful for predicting faults. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Machine sound can provide information about machine health. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Maintenance history is important for predicting future faults. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Preventive maintenance reduces unexpected machine breakdown. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Continuous machine monitoring would improve maintenance activities. | (✓) | ( ) | ( ) | ( ) | ( ) |
| An intelligent predictive maintenance system would reduce machine downtime. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Sensor-based monitoring would improve fault detection. | (✓) | ( ) | ( ) | ( ) | ( ) |
| Automated alerts would help maintenance personnel respond faster. | ( ) | (✓) | ( ) | ( ) | ( ) |
| An edge-based system would be useful in an industrial environment. | ( ) | (✓) | ( ) | ( ) | ( ) |

#### SECTION O: OPEN-ENDED QUESTIONS

- What are the most common causes of machine failure in your company?

## Bearing lubrication failures, linear wear, gearbox faults, motor overload and blocked diaphragms.

- What changes do you normally observe shortly before a machine develops a serious fault?

## Bearing temperature and vibration rise slowly, oil pressure falls, and motor current goes up.

- Are there particular sounds that indicate a developing fault? Please describe them.

## Sharp knocking, grinding from the girth gear and a high whine from the gearbox or bearings.

- What machine parameters are most useful for predicting failure?

## Bearing temperature, vibration, lube oil pressure, motor current and mill load.

- What challenges do you face with the current maintenance system?

## Data is in SCADA but not used for prediction and vibration is checked by hand.

- What features would you like to see in a predictive maintenance system?

## Early warnings several weeks ahead, a trend dashboard, and links to the maintenance history.

- Other information that may help in developing the expert system:

## Mill load and feed changes affect readings, so the system should adjust for them.