SECTION A: RESPONDENT INFORMATION

| 1. Position/Job Role | Maintenance Technician (sample response) |
| --- | --- |
| 2. Years of experience | 7–10 years |
| 3. Department/Unit | Utilities |
| 4. Type of industry/company | Food Processing / Carbonated Soft Drink Bottling |
| 5. How often do you interact with the machine? | Daily |

SECTION B: MACHINE IDENTIFICATION AND GENERAL INFORMATION

| 1. Machine name/type | Screw Air Compressor |
| --- | --- |
| 2. Machine identification/asset number | SU-UT-CMP-01 (sample asset ID – verify from plant asset register) |
| 3. Manufacturer | Imported bottling-equipment OEM (sample) |
| 4. Model number | SC-110 (sample) |
| 5. Serial number | SN-2012-0189 |
| 6. Date/year of manufacture | 2012 |
| 7. Date/year installed/commissioned | 2013 |
| 8. Age | More than 10 years |
| 9. Locally manufactured or imported? | Imported |
| 10. Main function | Supplies compressed air to run the pneumatic parts of the filler, packer, inspector and control valves. |
| 11. Normal operating capacity | About 18 m³ per minute at 8 bar (representative; verify plant record) |
| 12. Normal operating hours per day | 22–24 hours (representative utility schedule; verify plant record) |
| 13. Operating days per week | 6 days (representative; verify plant schedule) |

SECTION C: MACHINE OPERATING PARAMETERS

| Parameter | Monitored? | How measured / typical observation |
| --- | --- | --- |
| Temperature | Yes | Air-end discharge temperature on controller |
| Vibration | Yes | Portable vibration sensor during condition checks |
| Pressure | Yes | Discharge pressure transmitter and gauge |
| Speed/RPM | No | Not applicable – fixed-speed motor |
| Voltage | Yes | Panel voltmeter / controller |
| Current | Yes | Panel ammeter / controller |
| Power consumption | Yes | Energy meter on panel |
| Oil level | Yes | Sight glass on oil separator |
| Oil pressure | Yes | Oil-pressure gauge on controller |
| Fuel consumption | No | Not applicable – electric machine |
| Hydraulic pressure | No | No hydraulic subsystem fitted |
| Noise/sound | No | Not monitored; operator listening only |
| Load | Yes | Load/unload hours on controller |
| Flow rate | No | Not monitored on this machine |
| Other | Yes | Dew point, oil-separator differential pressure, running hours |

| 2. Normal operating temperature | About 80 °C (air-end discharge) |
| --- | --- |
| 3. Temperature at which abnormal behaviour begins | About 105 °C |
| 4. Normal operating speed/RPM | Not applicable – fixed-speed (motor 2,970 RPM) |
| 5. Normal operating pressure | Discharge about 7–8 bar |
| 6. Normal vibration level, if known | About 4 mm/s RMS expected for a healthy machine – investigate a sustained rise above baseline. |
| 7. Other important machine parameters | Dew point, oil-separator differential pressure, running hours. |
| 8. Are parameters recorded electronically? | Yes – compressor controller. |
| 9. If yes | Controller data; logbook for major events. |

SECTION D: MACHINE MAINTENANCE PRACTICES

| 1. Main maintenance type | Preventive |
| --- | --- |
| 2. Routine maintenance frequency | Weekly; major service per manufacturer/plant schedule |
| 3. Recommended service interval | 4,000 operating hours |
| 4. Date of last major service | 02 August 2026 (sample) |
| 5. Date of next scheduled service | 02 February 2027 (sample) |
| 6. Maintenance activities | Oil and filter change; air-filter and separator checks; cooler cleaning; belt/coupling checks; drain valve checks; safety valve testing; electrical inspection. |
| 7. Who performs maintenance? | Company maintenance personnel |
| 8. Is a maintenance logbook kept? | Yes |
| 9. Are previous machine faults recorded? | Often |
| 10. Usefulness of maintenance history for diagnosis | Very useful |

SECTION E: MACHINE FAILURE AND FAULT HISTORY

| 1. Has the machine experienced a major failure? | Yes (sample response) |
| --- | --- |
| 2. Fault frequency | Occasionally (representative response; verify actual maintenance records) |
| 3. Most common faults | Oil-separator clogging; air-end bearing wear; cooler fouling; intake-filter blockage; motor overload. |
| 4. Component that fails most frequently | Oil separator / air-end |
| 5. Date of most recent failure | 25 June 2026 (sample) |
| 6. Cause of most recent failure, if known | Clogged oil separator raising temperature. |
| 7. Downtime after failure | 4–10 hours |
| 8. Action taken to restore the machine | Separator and oil replaced, cooler cleaned. |
| 9. Was the failure predictable? | Yes |
| 10. Warning signs observed | Rising air-end temperature, higher differential pressure, rising current. |

SECTION F: MACHINE BEHAVIOUR BEFORE FAILURE

| 1. Does the machine show warning signs before a fault? | Often |
| --- | --- |
| 2. Warning signs observed | Rising discharge temperature; rising separator differential pressure; higher current; oil carry-over. |
| 3. How long before failure do warning signs appear? | Several days |
| 4. Earliest indication of a developing fault | Discharge temperature slowly rising. |
| 5. Most reliable indication of impending failure | Rising temperature with separator differential-pressure rise. |
| 6. Can an experienced operator detect an upcoming fault through observation? | Yes |
| 7. What does the operator normally observe? | Temperature, pressure, oil in air, noise. |

SECTION G: MACHINE SOUND AND NOISE BEHAVIOUR

| 1. Does the machine produce a different sound when developing a fault? | Yes |
| --- | --- |
| 2. Abnormal sound | Whining; rattling; screeching |
| 3. Component producing abnormal sound | Air-end and motor bearings |
| 4. Describe the normal machine sound | Steady compressor hum. |
| 5. Describe the sound immediately before failure | Rising whine with rattling. |
| 6. Does the abnormal sound increase gradually? | Yes |
| 7. Can an experienced operator identify a fault from sound? | Yes |
| 8. Usefulness of automatic sound monitoring | Very useful |

SECTION H: VISUAL AND PHYSICAL MACHINE CHANGES

| Observation | Frequency |
| --- | --- |
| Excessive vibration | Often |
| Increased temperature | Often |
| Oil leakage | Sometimes |
| Smoke | Rarely |
| Burning smell | Sometimes |
| Unusual noise | Sometimes |
| Reduced speed | Rarely |
| Reduced output | Sometimes |
| Increased energy consumption | Often |
| Irregular movement | Rarely |
| Frequent machine stoppage | Sometimes |

SECTION I: FAULT DETECTION AND DIAGNOSIS

| 1. Fault detection method | Sensors; controller alarms; operator observation |
| --- | --- |
| 2. Fault cause determination | Sensor readings; operator experience; technician inspection |
| 3. Normal diagnosis time | 30 min–1 hour |
| 4. Challenges | Lack of historical analysis; sensor limits. |

SECTION J: USE OF SENSORS AND SMART MONITORING

| 1. Does the machine have condition-monitoring sensors? | Yes – temperature, pressure and current sensors |
| --- | --- |
| 2. Sensor types | Temperature; pressure; current; dew point |
| 3. Are sensor readings stored for future analysis? | Yes – controller memory |
| 4. Measurement frequency | Continuously for control variables; alarm logging |
| 5. Would the company be willing to use sensors for continuous machine-health monitoring? | Yes |

SECTION K: PREDICTIVE MAINTENANCE EXPERT SYSTEM

| 1. Does the company currently use a predictive maintenance system? | No |
| --- | --- |
| 2. Would an intelligent system that predicts possible failure be useful? | Very useful |
| 3. Information the system should monitor | Discharge temperature; vibration; differential pressure; current; fault history. |
| 4. Preferred alert | Dashboard + mobile notification |
| 5. Preferred warning time | Several days |
| 6. Would you trust automated maintenance recommendations? | Only with human confirmation |
| 7. Final maintenance decision should be made by | System recommendation + human approval |

SECTION L: EDGE-BASED MONITORING

| 1. Prefer data processed locally near machine? | Yes |
| --- | --- |
| 2. Advantages | Faster response; continuous monitoring. |
| 3. Internet/network reliability | Reliable |
| 4. Should the system continue when internet is unavailable? | Yes – local edge processing should continue and synchronise data when connectivity returns. |

SECTION M: MAINTENANCE RECORD DATA – SAMPLE ENTRIES

| Date | Parameter / Observation | Fault Suspected | Cause | Maintenance Done | Parts Replaced | Downtime |
| --- | --- | --- | --- | --- | --- | --- |
| 25 / 06 / 2026 | Discharge temperature high | Clogged oil separator | Operating hours exceeded | Separator and oil replaced | Oil separator, oil | 7 h |
| 08 / 03 / 2026 | Air-end noise | Bearing wear | Normal ageing | Bearing replaced | Air-end bearing | 10 h |

SECTION N: OVERALL ASSESSMENT

Scale: SA = Strongly Agree; A = Agree; U = Undecided; D = Disagree; SD = Strongly Disagree.

| Statement | Rating |
| --- | --- |
| Machine failures can often be detected before complete breakdown. | SA |
| Machine operating parameters are useful for predicting faults. | SA |
| Machine sound can provide information about machine health. | A |
| Maintenance history is important for predicting future faults. | SA |
| Preventive maintenance reduces unexpected machine breakdown. | SA |
| Continuous machine monitoring would improve maintenance activities. | SA |
| An intelligent predictive maintenance system would reduce machine downtime. | SA |
| Sensor-based monitoring would improve fault detection. | A |
| Automated alerts would help maintenance personnel respond faster. | SA |
| An edge-based system would be useful in an industrial environment. | A |

SECTION O: OPEN-ENDED QUESTIONS

| 1. Most common causes of machine failure | Separator clogging, bearing wear, cooler fouling, overheating. |
| --- | --- |
| 2. Changes observed before serious fault | Temperature rise, pressure drop across separator, current rise. |
| 3. Sounds indicating developing fault | Whining or rattling from the air-end. |
| 4. Most useful parameters for predicting failure | Discharge temperature, vibration, separator differential pressure. |
| 5. Challenges with current maintenance system | Sensor limits, heat and dust. |
| 6. Desired predictive maintenance system features | Overheating alerts, service-life prediction. |
| 7. Other information useful for the expert system | Ambient heat and filter condition affect compressor life. |