### SECTION A: RESPONDENT INFORMATION

- Position/Job Role: Mechanical Engineer

- Years of experience:7-10 years

- Department/Unit:

## Milling Department

- Type of industry/company: Food Processing

- How often do you interact with the machine? Daily

### SECTION B: MACHINE IDENTIFICATION AND GENERAL INFORMATION

- Machine name/type:

## Wheat roller milling line for Semolina (Semoulita) production

- Machine identification/asset number:

## /M-002  (Sample)

- Manufacturer:

## /xample milling machinery Ltd (Sample)

- Model number:

## RM-600 (Sample)

- Serial number (if applicable):

## SM-2011-0642 (Sample)

- Date/year of manufacture:

## 2011

- Date/year machine was installed/commissioned:

## 2012

- Age:11-15 years

- Locally manufactured or imported?Imported

- Main function of the machine:

## Cleans and conditions wheat, then grinds and sifts it into Semolina (Semoulita)

and flour for packaging.

- Normal operating capacity:

## About 12 tonnes of wheat per hour (about 8 tonnes of Semolina per hour).

- Normal operating hours per day:

[ ] <4 [ ] 4-8 [ ] 9-12 [ ] 13-16 [ ] >16

- Operating days per week: 7

### SECTION D: MAINTENANCE PRACTICES (continued)

- Maintenance activities:

[✓] Lubrication [✓] Oil change [✓] /ilter replacement [✓] Bearing inspection [✓] Belt inspection/replacement [✓] /lectrical inspection [✓] Cleaning [✓] Calibration [✓] Tightening [✓] Replacement of worn parts

- Who performs maintenance?

[ ] Operator [ ] Company maintenance personnel [ ] External technician [ ] Manufacturer [✓] Combination

- Is a maintenance logbook kept?

[✓] Yes [ ] No

- Are previous machine faults recorded?

[✓] Always  [ ] Often  [ ] Sometimes  [ ] Rarely  [ ] Never

- Usefulness of maintenance history for diagnosis:

[✓] Very useful [ ] Useful  [ ] Moderate  [ ] Slight  [ ] Not useful

### SECTION E: MACHINE FAILURE AND FAULT HISTORY

- Has the machine experienced a major failure?

[✓] Yes [ ] No [ ] Don't know

- Fault frequency:

[ ] Very frequently [ ] Frequently [✓] Occasionally [ ] Rarely

- Most common faults:

## Roller bearing overheating, worn rolls, belt slip, Plansifter imbalance, blocked

sieves

- Component that fails most frequently:

## Roller bearings and drive belts

- Date of most recent failure:

## 23/09/2026

- Cause of most recent failure, if known:

## A worn roller bearing overheated and caused a roll to run unevenly.

- Downtime after failure:

[ ] <1 hour [ ] 1-4 hours [ ] 5-8 hours [✓] 9-24 hours [ ] >24 hours

- Action taken to restore the machine:

## Stopped the mill, replaced the bearings, re-gapped and realigned the rolls.

- Was the failure predictable?

[✓] Yes [ ] No [ ] Not sure

- Warning signs observed:

## Bearing temperature rose over several days, output fell, and the machine made a

rumbling noise.

### SECTION F: MACHINE BEHAVIOUR BEFORE FAILURE

- Does the machine show warning signs before a fault?

[ ] Always  [✓] Often [ ] Sometimes  [ ] Rarely  [ ] Never

- Warning signs observed:

[✓] Increased temperature [ ] Excessive vibration [ ] Unusual sound/noise [ ] Reduced speed [ ] Increased power consumption [✓] Increased current [ ] Pressure changes [ ] Smoke [ ] Burning smell [ ] Oil

leakage [✓] Reduced production output [ ] Irregular movement [ ] Excessive heating [ ] Intermittent stopping [ ] Alarm light [ ] Error message/code [ ] Other

- How long before failure do warning signs appear?

[ ] Minutes  [ ] Hours  [ ] 1 day  [✓] Several days [ ] Weeks  [ ] No fixed period

- Earliest indication of a developing fault:

## Bearing temperature slowly rises and the roller stand vibrates more.

- Most reliable indication of impending failure:

## Rising bearing temperature together with higher motor current.

- Can an experienced operator detect an upcoming fault through observation?

[✓] Yes [ ] No [ ] Sometimes

- What does the operator normally observe?

## Bearing temperature, belt noise, fall in extraction rate, flour dust and burning

smell.

### SECTION G: MACHINE SOUND AND NOISE BEHAVIOUR

- Does the machine produce a different sound when developing a fault?

[✓] Yes [ ] No [ ] Sometimes [ ] Not sure

- Abnormal sound:

[✓] Grinding [✓] Knocking [ ] Clicking  [✓] ffumming [✓] Screeching [✓] Squealing [✓]

Rattling [ ] Whining [ ] Hissing [ ] Popping [✓] Increased general noise [ ] Other

- Component producing abnormal sound:

## Roller bearings, drive belts, Plansifter, fans

- Describe the normal machine sound:

## A steady, even hum of the rollers with a soft swooshing from the sifter.

- Describe the sound immediately before failure:

## A loud rumble and grinding from the roller stand, and squealing from the slipping

belt.

- Does the abnormal sound increase gradually?

[✓] Yes [ ] No [ ] Sometimes

- Can an experienced operator identify a fault from sound?

[✓] Yes [ ] No [ ] Sometimes

- Usefulness of automatic sound monitoring:

[✓] Very useful [ ] Useful  [ ] Moderate  [ ] Slight  [ ] Not useful

### SECTION H: VISUAL AND PHYSICAL MACHINE CHANGES

| Observation | Never | Rarely | Sometimes | Often | Always |
| --- | --- | --- | --- | --- | --- |
| /xcessive vibration |  |  |  | ✓ |  |
| Increased temperature |  |  |  | ✓ |  |
| Oil leakage |  | ✓ |  |  |  |
| Smoke |  | ✓ |  |  |  |
| Burning smell |  |  | ✓ |  |  |
| Unusual noise |  |  |  | ✓ |  |
| Reduced speed |  |  | ✓ |  |  |
| Reduced output |  |  |  | ✓ |  |
| Increased energy consumption |  |  | ✓ |  |  |
| Irregular movement |  |  | ✓ |  |  |
| /requent machine stoppage |  |  | ✓ |  |  |

### SECTION I: FAULT DETECTION AND DIAGNOSIS

- Fault detection method:

[✓] Operator observation [✓] Alarm system [ ] Sensor [✓] PLC/SCADA [ ] Machine stops [ ] Technician diagnosis [ ] Routine inspection [ ] Other

- Fault cause determination:

[✓] Operator experience [✓] Maintenance manual [✓] Technician inspection [ ] Diagnostic software [✓] Sensor readings [✓] Trial and error [✓] Manufacturer support [ ] Other

- Normal diagnosis time:

[ ] <30 minutes [ ] 30 min-1 hour [✓] 1-3 hours [ ] 4-8 hours [ ] >8 hours

- Challenges:

[✓] Lack of sensors [ ] Lack of maintenance records [✓] Lack of skilled personnel [ ] Machine complexity [✓] Lack of diagnostic equipment [✓] Lack of historical data [✓] Poor documentation [ ] Other

### SECTION J: USE OF SENSORS AND SMART MONITORING

- Does the machine have condition-monitoring sensors?

[✓] Yes [ ] No [ ] Don't know

- Sensor types:

[✓] Temperature [✓] Vibration [✓] Pressure [✓] Current [ ] Voltage [✓] Speed [ ] Sound/acoustic [ ] Humidity [ ] Oil condition [ ] Other

- Are sensor readings stored for future analysis?

[ ] Yes  [✓] No [ ] Don't know

- Measurement frequency:

[✓] Continuously [ ] Every second [ ] Every minute [ ] Every 5-10 minutes [ ] Hourly [ ] Daily [ ] Manual

- Would the company be willing to use sensors for continuous machine-health monitoring?

[✓] Yes [ ] No [ ] Maybe

### SECTION K: PREDICTIVE MAINTENANCE EXPERT SYSTEM

- Does the company currently use a predictive maintenance system?

[ ] Yes [ ] No [✓] Don't know

- Would an intelligent system that predicts possible failure be useful?

[✓] Very useful [ ] Useful  [ ] Moderate  [ ] Slight  [ ] Not useful

- Information the system should monitor:

[✓] Temperature [✓] Vibration [✓] Pressure [✓] Current [✓] Voltage [✓] Speed [✓] Machine sound [✓] Operating hours [✓] Maintenance history [ ] Machine age [✓] /ault history [✓] Production output

- Preferred alert:

[ ] Computer warning [ ] Mobile notification [ ] SMS [ ] Sound alarm [ ] Dashboard [ ] Email [✓] Combination

- Preferred warning time:

[ ] Minutes  [ ] Hours  [ ] 1 day  [✓] Several days  [ ] Several weeks

- Would you trust automated maintenance recommendations?

[ ] Yes  [ ] No [✓] Only with human confirmation

- Final maintenance decision should be made by:

[ ] System  [ ] Maintenance personnel  [ ] Operator  [✓] System recommendation + human approval

### SECTION L: EDGE-BASED MONITORING

- Would the company prefer machine data to be processed locally near the machine?

[✓] Yes [ ] No [ ] Not sure

- Advantages:

[✓] /aster fault detection [✓] Reduced internet dependence [ ] Improved data privacy [✓] /aster response [✓] Reduced data transmission [✓] Continuous monitoring [ ] Other

- Internet/network reliability:

[ ] Very reliable  [ ] Reliable  [✓] Sometimes unreliable  [ ] Frequently unreliable  [ ] Not available

- Should the system continue operating when internet connectivity is unavailable?

[✓] Yes [ ] No [ ] Not sure

### SECTION M: MAINTENANCE RECORD DATA

| Date | Parameter / Observation | Fault | Suspected Cause | Maintenance Done | Parts Replaced | Downti me |
| --- | --- | --- | --- | --- | --- | --- |
| 23/09/20 26 | Roller bearing at 68: rumbling noise, output down | Bearing o verheating | Worn bearing, low grease | Bearing replaced, rolls re-gapped and realigned | Roller bearing | 16 hours |
| 05/08/20 26 | Sifter shaking more, poor separation | Plansifter imbalanc e | Loose counter weight, worn drive belt | Rebalanced and tightened | Drive belt | 5 hours |
| 30/05/20 26 | Motor current high, semolina output fall | Choked conveying line | Dust build-up and blocked sieves | Lines cleaned, sieves replaced | Sieve cloth | 4 hours |

### SECTION N: OVERALL ASSESSMENT

Scale: SA = Strongly Agree; A = Agree; U = Undecided; D = Disagree; SD = Strongly Disagree.

| Statement | SA | A | U | D | SD |
| --- | --- | --- | --- | --- | --- |
| Machine failures can often be detected before complete breakdown. | ✓ |  |  |  |  |
| Machine operating parameters are useful for predicting faults. | ✓ |  |  |  |  |
| Machine sound can provide information about machine health. | ✓ |  |  |  |  |
| Maintenance history is important for predicting future faults. | ✓ |  |  |  |  |
| Preventive maintenance reduces unexpected machine breakdown. | ✓ |  |  |  |  |
| Continuous machine monitoring would improve maintenance activities. | ✓ |  |  |  |  |
| An intelligent predictive maintenance system would reduce machine downtime. |  | ✓ |  |  |  |
| Sensor-based monitoring would improve fault detection. | ✓ |  |  |  |  |
| Automated alerts would help maintenance personnel respond faster. |  | ✓ |  |  |  |
| An edge-based system would be useful in an industrial environment. |  | ✓ |  |  |  |

### SECTION O: OPEN-ENDED QUESTIONS

- What are the most common causes of machine failure in your company?

## Worn rolls and bearings, belt slip, blocked sieves, dust build-up, poor lubrication

and moisture problems in the wheat.

- What changes do you normally observe shortly before a machine develops a serious fault?

## Bearing temperature and vibration rise, motor current goes up, and semolina

output and extraction rate drop.

- Are there particular sounds that indicate a developing fault? Please describe them.

## A rumble or grinding from the roller stand and a squeal from slipping belt.

- What machine parameters are most useful for predicting failure?

## Bearing temperature, vibration, motor current, roll gap and extraction rate.

- What challenges do you face with the current maintenance system?

## Vibration and temperature are checked by hand, and records are on paper.

- What features would you like to see in a predictive maintenance system?

## /arly warning several days ahead, trend charts and service reminders based on

running hours.

- Other information that may help in developing the expert system:

## Wheat quality and moisture affect the readings, so the system should record the

wheat batch and tempering moisture.